台灣科技大學 (115) 學年度 Homework 1 Due Date: 115 年 11 月 30 日

科目: 超大型積體電路測試與可測試設計

Problem 1: Fault Modeling & Collapsing

Consider a 3-input AND gate with inputs A, B, C, and output Z.

(a) List all the single stuck-at faults for this gate. (3 pts)

- Apply fault equivalence and fault dominance reduction techniques to collapse the initial fault set. Show your step-by-step reasoning and state the final minimum collapsed fault Set. (12 pts)

Problem 2: Transistor-Level Fault Models

Consider a 3-input NAND gate with inputs A, B, C and output Z.

- Draw the CMOS circuit diagram for the three-input NAND gate. (3 pts)

- Determine the number of transistor faults for both single-fault and multiple-fault assumptions. (6 pts)

- How can you test for stuck-open and stuck-on faults in the NAND gate. Provide examples to illustrate your method. (6 pts)

Problem 3: Scan Design

- Explain how full-scan design transforms the complexity of Sequential ATPG to combinational ATPG. Why is this transformation a necessity for VLSI testing? (5 pts)

- Describe a complete operating sequence for a single scan test pattern. Detail what happens in the circuit during the shift-in phase, capture phase, and shift-out phase. Draw or write out the precise timing behavior of the scan enable (SE) signal relative to the system clock (CLK) across these phases. (15 pts)

Problem 4: Test Time of Scan-Based Design

The major drawback of scan-based Design-for-Testability (DFT) is the significant increase in test application time due to the serial shifting mechanism. For a single test pattern, the automatic test equipment (ATE) must serial shift the test stimulus into the scan chain, trigger a functional capture cycle, and serial shift the response out. For a design with Nv test patterns, the exact formula for the total number of clock cycles required for a single scan chain containing Nff flip-flops is:

Total Cycles = (Nff  Nv) + Nff + Nv

Nff  Nv represents the bits shifted in for Nv patterns.

Nff represents the final shift-out cycle needed to unload the response of the last Nv-th pattern. The last Nv represents the capture clock cycles (one capture cycle per pattern). For a multiple scan chain configuration with K parallel scan chains, assuming the flip-flops are balanced evenly, the longest chain length determines the shift time:

Nmax_chain =  Nff /K

The formula updates to:

Total Cycles = (Nmax_chain  Nv) + Nmax_chain + Nv

We assume that an industrial ASIC design contains 60,000 flip-flops (Nff = 60,000). The ATPG tool generates 4,000 test patterns (Nv = 4,000) to achieve the targeted stuck-at fault coverage. The ATE runs the scan shift operations at a frequency of 25 MHz, while the functional capture cycles are executed at 200 MHz.

- Assume all 60,000 flip-flops are stitched into a single, long scan chain. Calculate the total number of clock cycles required to execute the entire test suite. (5 pts)

- Calculate the actual total test time (in seconds). (Hint: Keep track of which cycles run at the shift frequency vs. the capture frequency). (5 pts)

- To reduce the manufacturing test cost, the DFT engineer reconfigures the design into 16 parallel scan chains (K = 16), assuming the flip-flops are distributed perfectly evenly among the chains. Determine the length of the longest scan chain (Nmax_chain). (2 pts)

- Calculate the new total test time (in seconds) under this 16-chain balanced configuration. (3 pts)

Problem 5: Test Generation

For the circuit shown in Fig. 1, derive test patterns for detecting x2 stuck-at 0 and signal 8 stuck at 1. (10 pts)

Fig. 1

Problem 6: Parallel Fault Simulation

Assuming a four-bit machine word, demonstrate parallel fault simulation of vector (x1, x2, x3,x4) = (0, 1, 0, 1) for x1/1, 5/1, and 10/0 faults in the shown in Fig. 1 circuit. (15 pts)

Problem 7: Deductive Fault Simulation (15%)

Consider the vectors (1, 1) and (0, 1) applied to the circuit shown in Fig. 2. We will simulate the s-a-0 and s-a-1 faults on all lines a through g for these vectors using deductive fault simulation. Determine which faults can be detected using these two test patterns. (10 pts)

Fig. 2

1.(a)

A SA0

A SA1

B SA0

B SA1

C SA0

C SA1

Z SA0

Z SA1

1.(b)

Step 1：使用 Fault Equivalence（故障等價）化簡

若兩個故障在所有可能的輸入組合下，都會產生相同的錯誤輸出行為，則稱為 Fault Equivalent（故障等價），可以只保留其中一個代表故障。

觀察 SA0 故障

當 \(A\) 發生 SA0 時：

\[ Z=0\cdot B\cdot C=0 \]

不論輸入 \(B、C\) 是多少，輸出都固定為 0。

同理：

\(A/0\)：輸出永遠為 0。

\(B/0\)：輸出永遠為 0。

\(C/0\)：輸出永遠為 0。

\(Z/0\)：輸出永遠為 0。

因此，這四個故障彼此等價：

\[ \boxed{A/0\equiv B/0\equiv C/0\equiv Z/0} \]

保留一個代表即可，例如 \(Z/0\)。

原本 8 個故障經過等價化簡後，剩下：

\[ F_{\text{eq}}=\{Z/0,A/1,B/1,C/1,Z/1\} \]

故障數由 8 個減少至 5 個。

Step 2：使用 Fault Dominance（故障支配）化簡

故障支配的核心概念是：如果測試某個故障所使用的測試向量，也必定能偵測另一個故障，則後者可以在故障覆蓋的角度被移除。

先找出各個 SA1 故障的測試向量。

對於輸入 \(A/1\)，必須讓正常電路輸出為 0，而故障電路輸出為 1。

選擇：

\[ (A,B,C)=(0,1,1) \]

正常電路：

\[ Z=0\cdot1\cdot1=0 \]

故障電路（\(A\) 被固定為 1）：

\[ Z_f=1\cdot1\cdot1=1 \]

所以測試向量 \(011\) 可以偵測 \(A/1\)。

同理可得：

| 故障 | 測試向量 \(ABC\) | 正常輸出 \(Z\) | 故障輸出 \(Z_f\) |
| --- | --- | --- | --- |
| \(A/1\) | 011 | 0 | 1 |
| \(B/1\) | 101 | 0 | 1 |
| \(C/1\) | 110 | 0 | 1 |

接著分析輸出故障 \(Z/1\)。

只要正常輸出 \(Z=0\)，就能偵測 \(Z/1\)，因為故障輸出會固定為 1。

因此，上述三個測試向量 \(011、101、110\) 也都能偵測 \(Z/1\)。

也就是說：

\[ T(A/1)\subseteq T(Z/1) \]

\[ T(B/1)\subseteq T(Z/1) \]

\[ T(C/1)\subseteq T(Z/1) \]

其中 \(T(f)\) 代表能偵測故障 \(f\) 的測試向量集合。

因此，保留 \(A/1、B/1、C/1\) 時，\(Z/1\) 已可由這些故障的測試向量一併偵測，不必再單獨保留 \(Z/1\)。

移除 \(Z/1\) 後，得到最終化簡集合。

Step 3：最終 Minimum Collapsed Fault Set

\[ \boxed{F_{\text{collapsed}}=\{Z/0,A/1,B/1,C/1\}} \]

2.(a)

2.(b)

單一故障假設： 6 + 6 = 12

多重故障假設：

每個電晶體在物理上存在 3 種互斥狀態：3^6 - 1 = 728

- 正常（Fault-free）

- Stuck-Open

- Stuck-On

2.(c)

Stuck-open fault：

由於 CMOS 電路可能保留先前的輸出電荷，因此需要利用兩個連續的測試向量來偵測。

當輸入由 \(000\) 變成 \(111\) 時，正常電路的輸出應由 1 變為 0。若串聯 NMOS 中有一顆 stuck-open，放電路徑會中斷，輸出可能保持在 1。

Stuck-on fault：

當輸入A,B,C=0,1,1，正常情況下，A 控制的 NMOS 應截止，但如果該 NMOS 持續導通，另外兩顆 NMOS 又因 B、C 為 1 而導通，便可能形成從輸出到 GND 的放電路徑，使輸出由預期的 1 變成 0，從而偵測故障。

3.(a)

將所有FF改為scan FF並將其串為scan chain，scan chain讓 ATPG 可以直接透過scan in觀察scan out並得到內部狀態訊息。

降低 ATPG 複雜度、提高可控制性、提高可觀察姓、提高故障覆蓋率

3.(b)

1. Shift-in phase（移入測試資料）

- 將 SE 設為 1，啟用 Scan mode。

- 每個有效的 CLK 時脈邊緣，測試資料透過 Scan-In（SI）逐位移入 Scan Chain。

- 原本儲存在 Scan FF 中的資料會逐級移動，直到完整的測試向量載入所有 Scan FF。

- 此階段主要用於設定內部電路狀態，不是執行正常功能運算。

2. Capture phase（擷取測試反應）

- 在 Shift-in 完成後，將 SE 設為 0，切換至 Normal mode。

- 施加一個或多個有效的 CLK 時脈邊緣，讓組合邏輯根據載入的測試狀態與外部輸入進行運算。

- Scan FF 擷取組合邏輯的輸出，將測試反應儲存起來。

3. Shift-out phase（移出測試反應）

- 將 SE 設回 1，重新啟用 Scan mode。

- 每個有效的 CLK 時脈邊緣，將 Scan FF 內儲存的測試反應逐位移出至 Scan-Out（SO）。

- 同時，新的資料也可能從 SI 移入。

- 將讀出的結果與預期的正確結果（Expected Response）比較，即可判斷是否存在故障。

Shift-in          Capture          Shift-out

CLK   ______/‾\_/‾\_/‾\_/‾\____/‾\______/‾\_/‾\_/‾\_/‾\____

SE    ‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾\___________/‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾‾

1 = Scan mode                    0                  1 = Scan mode

4.(a)

Total Cycles&=60,000×4,000&+60,000+4,000&=240,000,000+60,000+4,000&=240,064,000

4.(b)

計算 Shift 時間

Cs&=NffNv+Nff&=60,000×4,000+60,000&=240,060,000

Shift 頻率為 25 MHz，即每秒 25,000,000 個週期。

Ts&=Csfs&=240,060,00025×106&=9.6024秒

計算 Capture 時間

Cc=Nv=4,000

Capture 頻率為 200 MHz。

Tc&=Ccfc&=4,000200×106&=0.00002 秒

計算總測試時間

Ttotal&=Ts+Tc&=9.6024+0.00002&=9.60242 秒

4.(c)

Nmax_chain=NffK

Nmax_chain=60,00016=3,750

4.(d)

Tshift=NvNmax_chain+Nmax_chainfs
=4,0003,750+3,75025×106=0.60015s

Capture 時間不變：

Tcapture=Nvfc=4,000200×106=0.00002 s

Ttotal&=Tshift+Tcapture&=0.60015+0.00002&=0.60017seconds

5.

x2 stuck-at 0

1. 故障激發（Fault Activation）

欲激發信號 2 (x2) 的 s-a-0 故障，必須將正常邏輯值設為 1：

- 設定 x2 = 1（此時信號 2 呈現 D 狀態，即 1/0）。

2. 故障傳播至扇出點（Propagation to Node 5）

欲使信號 2 的 D 通過 AND 閘 G1 傳播至信號 5，G1 的另一輸入端必須設為非控制值（Non-controlling value）1：

- 設定 x3 = 1（信號 3 = 1）

- 此時信號 5 = D，扇出分支信號 6 = D、信號 7 = D。

3. 選擇並敏化傳播路徑至主要輸出（信號 11）

信號 5 分支為兩條路徑（經由 G3 或 G4），需分析哪一條路徑可將故障效應成功傳播至輸出端 11：

- 嘗試路徑 A（經由 G3 -> 信號 9 -> G5）：

  - 欲使信號 6 (D) 通過 NOR 閘 G3，需設信號 1 (x1) = 0，此時信號 9 = D'（即 0/1）。

  - 接著欲使信號 9 (D') 通過 OR 閘 G5，另一輸入信號 10 必須為 OR 閘的非控制值 0。

  - 然而，信號 10 是 NAND 閘 G4 的輸出。欲使信號 10 恆為 0，G4 的兩個輸入（信號 7 與信號 8）必須同時為 1。但目前信號 7 為 D（故障時為 0），在故障發生時信號 10 必定變為 1，這會遮蔽（Mask）信號 9 的故障效應。因此無法單獨透過路徑 G3 傳播。

- 選擇路徑 B（經由 G4 -> 信號 10 -> G5）：

  - 通過 NAND 閘 G4： 欲使信號 7 (D) 通過 G4，另一輸入端信號 8 必須設為 NAND 閘的非控制值 1。

    - 設定 信號 8 = 1 → 反推經由反相器 G2，設定 x4 = 0。

    - 此時信號 10 = (D NAND 1) = D'（即 0/1）。

  - 通過 OR 閘 G5： 欲使信號 10 (D') 通過 OR 閘 G5 到達輸出 11，另一輸入端信號 9 必須穩定為 OR 閘的非控制值 0（避免被同源扇出信號干擾）。

    - 信號 9 為 NOR 閘 G3 的輸出。只要將 G3 的輸入端信號 1 (x1) 設為 NOR 閘的控制值 1，不論信號 6 為何，信號 9 都會強制鎖定為 0。

    - 設定 x1 = 1 → 信號 9 = 0。

    - 此時主要輸出 信號 11 = (0 OR D') = D'（正常輸出為 0，故障輸出為 1），故障成功傳播至輸出！

- (註：若設 x1 = 0, x4 = 0，則信號 9 = D' 且信號 10 = D'，經過 OR 閘 G5 後，D' OR D' = D'，多重路徑敏化同樣成立！)

x2 s-a-0 測試向量結果

- (x1, x2, x3, x4) = (1, 1, 1, 0) 或 (0, 1, 1, 0)

signal 8 stuck-at-1

1. 故障激發（Fault Activation）与前向反推（Justification）

欲激發信號 8 的 s-a-1 故障，必須將信號 8 的正常邏輯值設為 0（使信號 8 呈現 D'，即 0/1）：

- 欲使信號 8 = 0，經由反相器 G2 反推，必須設定輸入 x4 = 1（信號 4 = 1）。

2. 故障傳播（Fault Propagation）

信號 8 僅有一條通往輸出端 11 的路徑（信號 8 -> G4 -> 信號 10 -> G5 -> 信號 11）：

- 通過 NAND 閘 G4（傳播至信號 10）：

  - 欲使信號 8 (D') 通過 NAND 閘 G4，另一輸入端 信號 7 必須設為非控制值 1。

  - 由於信號 7 來自信號 5（AND 閘 G1 的輸出），欲使信號 5 = 1，必須將 G1 的兩個輸入端皆設為 1：

    - 設定 x2 = 1 且 x3 = 1。

  - 此時信號 7 = 1，經過 G4 後，信號 10 = (1 NAND D') = D（即 1/0）。

- 通過 OR 閘 G5（傳播至信號 11）：

  - 欲使信號 10 (D) 通過 OR 閘 G5，另一輸入端 信號 9 必須為非控制值 0。

  - 檢查信號 9 的狀態：在前一步驟中，因為 x2 = 1 且 x3 = 1，使得信號 5 = 1，進而使分支 信號 6 = 1。

  - 由於信號 6 = 1（已為 NOR 閘 G3 的控制值），不論信號 1 (x1) 為 0 或 1，NOR 閘 G3 的輸出 信號 9 必定恆為 0：

    - x1 = X（Don't care，可為 0 或 1）。

  - 此時主要輸出 信號 11 = (0 OR D) = D（正常輸出為 1，故障輸出為 0），故障成功傳播至輸出！

3. signal 8 s-a-1 測試向量結果

- (x1, x2, x3, x4) = (X, 1, 1, 1)，即 (0, 1, 1, 1) 或 (1, 1, 1, 1)

6.

7.